# Induced Vibration from Rotating Machinery: Fundamentals and a Preliminary Load Method for a Momentum Wheel and Rotating Arm Assembly

## Abstract

Any machine with spinning parts pushes on the structure that holds it. The pushes are small, but they repeat with every revolution, and in precision systems they can be large enough to disturb pointing, blur sensors, or excite structural modes. These loads are known as induced vibration, and estimating them is a routine part of designing anything that carries rotating hardware.

The first part of this paper covers the fundamentals in general terms. It explains where induced loads come from, the physics and equations behind each source, how loads are described in a coordinate system, and how the field normally characterizes them before and after hardware exists.

The second part applies those ideas to our assembly, which consists of two momentum wheels and a rotating arm, each on its own stationary shaft with a pair of tapered roller bearings. With no test data available, a purely analytical method was developed that estimates the six load components Fx, Fy, Fz, Mx, My and Mz from CAD mass properties, the eccentricity specification, the operating speeds, and the bearing geometry, preload and specified imperfections. The paper describes the model, the choices made along the way and the reasons for them, and what the method does not cover. The resulting loads are conservative, hard-mounted estimates intended for early design, and they are meant to be refined as balance data, supplier data and test results become available.

## 1. Introduction

Rotating hardware such as momentum wheels, gimbals, pumps and cryocoolers shakes whatever it is attached to. This is called induced vibration. Even a rotor built to drawing and balanced to specification passes small forces and moments into its mount on every revolution, because no part is made perfectly: its center of mass sits slightly off the axis, it sits slightly crooked on its shaft, and its bearings and motor are not perfectly smooth. The loads are small, but they repeat at fixed frequencies, and near a structural resonance they can be amplified into jitter, sensor noise or fatigue. Because their size and frequency are set early by choices about speeds, balance tolerances and bearings, they need to be understood early in the design.

Our assembly contains two momentum wheels and a rotating arm, each on its own stationary shaft with tapered roller bearings and its own motor. The wheels spin opposite to the arm to cancel its angular momentum. The task was to determine the forces and moments these parts induce, including the bearing disturbances at harmonics of the arm and wheel speeds, before any hardware existed. The only inputs were CAD mass properties, the specified center of mass offsets, the operating speeds and profile, and bearing catalog data. This paper develops a method that estimates the resulting source loads, Fx, Fy, Fz, Mx, My and Mz, from that data alone, as they would act on a rigid mount.

Section 2 describes the sources of induced vibration and the equations behind each one. Section 3 describes our assembly, Section 4 defines the coordinate system and the loads, and Section 5 lists the model inputs. Section 6 sets out the model equations as applied to our assembly, and Section 7 explains the analysis method and the reasoning behind it. Section 8 covers assumptions and limitations, and Section 9 gives next steps and conclusions.

## 2. Induced Vibration Fundamentals

### 2.1 Where induced loads come from

A rigid body can only push on its supports with a force or a moment. Newton’s second law says it needs a net force whenever its center of mass accelerates, and Euler’s equation says it needs a net moment whenever its angular momentum changes in size or direction:

F=m acm	(1)

M=dHdt	(2)

A perfectly made rotor spinning at constant speed about one of its principal axes does neither. Its center of mass sits still on the axis, and its angular momentum points steadily along the shaft, so it transmits nothing to its mount beyond its own weight.

Real rotors fall short of that ideal in several ways, and each shortfall becomes a source of induced vibration. The rotor itself can be out of balance, either statically or dynamically. Its bearings can be imperfect. Its motor can deliver torque unevenly. Random processes inside the bearings and electronics can add noise. And if the rotor’s mount is itself turning, the rotor’s angular momentum produces gyroscopic moments. The sections below describe each of these in turn. Throughout, Ω is the spin rate in rad/s, N is the same speed in rev/min, and fr is the rotor frequency in Hz:

Ω=2πN60	(3)

fr=N60=Ω2π	(4)

### 2.2 Static imbalance

Static imbalance exists when the center of mass of a rotor does not lie on its axis of rotation. As the rotor turns, its center of mass is carried around a small circle on every revolution. Keeping a mass on a circular path requires a force toward the center, and the only thing that can supply it is the shaft. The rotor pulls back on the shaft with an equal force directed outward toward its heavy side.

The size of the effect is set by the product of the rotor mass m and the offset e of its center of mass, a quantity called the static imbalance Us. Since the center of mass accelerates toward the axis at eΩ², Equation (1) gives the force on the mount:

Us=m e	(5)

Fs=Us Ω2	(6)

The force rotates with the rotor and repeats once per revolution. Because it grows with the square of the spin rate, it is small at low speed and becomes the dominant source in most rotating machinery at operating speed.

Static imbalance gets its name because it can be detected without spinning anything. A statically unbalanced wheel set on frictionless rails will roll until its heavy side is at the bottom. It is corrected by adding or removing mass in a single plane.

The acceptable amount of static imbalance is usually set by a balance quality grade under ISO 21940. The grade number G is the product of the allowed center of mass offset and the operating angular speed:

G=e Ω mm/s	(7)

Precision spindles and gyroscopes are commonly balanced to G 1 or better, while general industrial machinery is often balanced to G 6.3. Because the grade ties the offset to the speed, faster rotors must be balanced more tightly to achieve the same grade.

### 2.3 Couple (dynamic) imbalance

A rotor can have its center of mass exactly on the shaft and still shake. If its principal axis of inertia is tilted slightly from the spin axis, the rotor’s angular momentum no longer lines up with the shaft. As the rotor turns, the angular momentum sweeps around a narrow cone, and by Equation (2) changing its direction in this way requires a moment. The shaft supplies it, and the rotor pushes back with a rocking moment that rotates with it, once per revolution.

This is couple imbalance. In wheel and vibration work it is often called dynamic imbalance; strictly, ISO 21940 uses “dynamic unbalance” for the general case of static and couple imbalance together, so “couple” is used here for the tilt part alone. For a small tilt θ, it depends on the difference between the inertia about the spin axis, Izz, and the inertia about the sideways axis the rotor is tilted toward, I⊥:

Ud=Izz−I⊥ θ	(8)

Md=Ud Ω2	(9)

The inertia difference governs how strongly a rotor responds to a tilt. A flat disc, whose spin inertia is about twice its sideways inertia, produces a noticeable moment from a small tilt. A body with equal inertias in every direction produces none, however it is tilted. Long, slender rotors such as arms and booms can have very different inertias about their two sideways axes. When the direction of the tilt is not known, the larger of the two differences gives an upper bound:

Ud,max=maxIzz−Ixx, Izz−Iyyθ	(10)

Couple imbalance only shows itself when the rotor spins. Correcting it requires balancing in two planes along the rotor.

|  | Static imbalance | Couple (dynamic) imbalance |
| --- | --- | --- |
| Physical cause | Center of mass off the spin axis | Principal axis tilted from the spin axis |
| Measure | Us = m e | Ud = ¦Izz − I⊥¦ θ |
| Load on the structure | Rotating sideways force | Rotating rocking moment |
| Load components | Fx, Fy | Mx, My |
| Frequency | Once per revolution | Once per revolution |
| Detectable at rest | Yes | No |
| Corrected by | Single-plane balancing | Two-plane balancing |

Real rotors almost always carry both kinds at once. They come from different manufacturing errors, so the angle between the heavy spot and the direction of tilt is effectively random and is rarely known before the rotor is balanced.

Imbalance as a specified limit. Before hardware exists, the actual imbalance of a part cannot be known: CAD and drawings describe the ideal part, and manufacturing errors are only revealed by measurement. Rotors of this kind are balanced after assembly, so what matters at the design stage is the imbalance the part is allowed to have once balanced. That limit is set by a balance specification, given directly as Us and Ud, as an eccentricity, or as an ISO 21940 balance grade G. With a grade, the allowed static imbalance follows from Equation (7):

Us=m e=m GΩ	(11)

When only a static limit is specified, the couple limit is commonly taken by applying the same tolerance in each of the two balancing planes, a distance b apart. The worst case is the two residuals pointing in opposite directions, which gives:

Ud=Us2 b	(12)

Loads calculated from these limits are the largest the balanced part can produce, and balancing to the same limits is verified on the hardware.

### 2.4 Bearing-induced vibration

Rolling element bearings are a separate source from the rotor itself. No bearing is perfectly smooth or perfectly round. Its races have small waves and high spots, its rolling elements are not all exactly the same size, and its cage does not run perfectly true. Each time a rolling element passes over one of these imperfections, the bearing gives the structure a small kick.

The timing of these kicks is fixed by the bearing’s geometry: the number of rolling elements Z, the element diameter D, the pitch diameter dp and the contact angle α. For a bearing whose outer race turns with the rotor at fr while the inner race is held still on a stationary shaft, the characteristic frequencies are:

fc=fr21+Ddpcosα	(13)

fo=Zfr21−Ddpcosα	(14)

fi=Zfr21+Ddpcosα	(15)

fb=dp2D fr1−Ddpcosα2	(16)

Equation (13) is the cage frequency, the rate at which the set of rolling elements travels around the bearing. Equation (14) is the outer race pass frequency, the rate at which elements pass a given point on the outer race. Equation (15) is the inner race pass frequency, the rate at which they pass a given point on the inner race. Equation (16) is the rolling element spin frequency. A defect on a rolling element strikes both races once per spin, so it often appears at twice fb. When the inner race turns instead and the outer race is fixed, the sign inside the cage frequency changes, giving (fr/2)(1 − (D/dp) cos α), while the two race pass frequencies and the spin frequency are unchanged. Two identities hold in either case and make useful checks:

fo+fi=Zfr	(17)

fo=Zfr−fc	(18)

None of these frequencies is a whole multiple of the shaft speed, which is how bearing disturbances are told apart from imbalance in measured spectra.

The frequencies can be calculated exactly from catalog data. The size of the disturbances depends on how large the imperfections are and how stiff the bearing is, and both can be estimated from geometry, preload and specified tolerances, as follows.

Contact stiffness from preload. An axial preload Fpre on an angular contact or tapered bearing is shared by the Z rolling elements, each carrying a load Q0 along its contact line. For roller (line) contacts, the deflection at each contact follows Palmgren’s empirical relation, in which δ is in millimeters, Q in newtons and the effective roller length l in millimeters. A roller has two contacts, one on each race, in series:

Q0=FpreZsinα	(19)

δc=3.84×10−5 Q00.9l0.8,  δ0=2 δc	(20)

Because the load grows as δ^(10/9), the stiffness of one roller about the preloaded state, and the resulting radial and axial stiffness of the whole bearing, are:

kr=dQdδ=109 Q0δ0	(21)

Krad=Z2 krcos2α	(22)

Kax=Z krsin2α	(23)

Which imperfections produce a net force. A race with waviness of order q (q lobes around its circumference) and amplitude A pushes each rolling element in or out by A cos(qψ), where ψ is the element’s angular position. The forces of all Z elements add up to a net load only for particular orders, because the sum over equally spaced elements cancels otherwise:

j=1Zcosqψjcosψj=Z2  if q=kZ±1,  j=1Zcosqψj=Z  if q=kZ	(24)

Orders q = kZ ± 1 therefore produce a lateral force and q = kZ an axial force, each of a size set by the contact stiffness. For a race:

Flat=Z2 krAcosα  each order kZ±1,  Fax=Z krAsinα  q=kZ	(25)

On a stationary race these lines fall at k times its race pass frequency. On a rotating race they fall at k times its race pass frequency plus or minus the shaft frequency for the lateral lines, and at k times the race pass frequency for the axial line. Waviness on the rolling elements matters for even orders q = 2k, which change the element diameter by 2A; it gives lateral lines at 2k fb ± fc and an axial line at 2k fb. An element that is oversize by Δ carries extra load and travels with the cage, giving a lateral force kr Δ cos α at the cage frequency. Because the elements sit at the contact angle, every lateral disturbance has an axial partner:

Fax=Flattanα	(26)

Bearings with a contact angle are therefore one of the few sources of axial force in a rotating assembly.

Force that reaches the structure. The forces above are “blocked” forces: what the bearing would apply if the rotor could not move. In reality the rotor sits on its bearings as a mass m on a spring of stiffness K, with a natural frequency fn. Well below fn the rotor simply follows the imposed displacement and only its inertia force reaches the shaft; well above fn the full blocked force does. With damping ratio ζ and r the ratio of the line frequency to fn:

fn=12πKm	(27)

Fshaft=Fblocked r21−r22+2ζr2,  r=ffn	(28)

Because bearing contacts are very stiff, fn is usually several kilohertz, far above the bearing frequencies, and the transmitted force is a small fraction of the blocked force. The model is linear about the preloaded state and is valid only while the imperfections are small compared with the preload deflection δ_0, so that no rolling element unloads.

### 2.5 Motor-induced vibration

The motor that drives a rotor produces its own disturbances, all of them acting mainly as twisting moments about the spin axis.

The largest is the reaction torque during speed changes. To accelerate the rotor, the motor applies a torque to it, and the motor’s stator, which is fixed to the structure, receives the same torque in the opposite direction. For a rotor accelerating at Ω̇, the torque on the mount is:

T=− Izz Ω	(29)

For a constant ramp from rest to Ω in time t, the acceleration is simply:

Ω=Ωt	(30)

This load lasts only as long as the speed is changing, but it can be much larger than the vibratory loads. In assemblies with several rotors, the rotors can be arranged to spin in opposite directions so that their reaction torques cancel.

A speed change also gives the static imbalance a small tangential component, Us Ω̇. Compared with the centripetal force of Equation (6), its size is:

FtFs=Us ΩUs Ω2=1Ω t	(31)

For ramps lasting several seconds this ratio is well under 1%, so the term is usually neglected.

On top of the reaction torque, motors produce small periodic variations in their output. Torque ripple comes from the way current is switched between windings. Cogging comes from the magnets being attracted to the steel teeth of the stator. For a three-phase motor with p pole pairs and Ns stator slots, they occur at:

fripple=6 n p fr	(32)

fcog=n LCMNs, 2pfr,  n=1,2,3,…	(33)

Both frequencies can be predicted from the motor’s design, but their amplitudes depend on the specific motor and are taken from datasheets or tests. An off-center rotor inside the motor can also create small radial magnetic forces.

### 2.6 Broadband noise

Not all induced vibration falls at distinct frequencies. Surface roughness in the bearings, irregular motion of the cage, lubricant behavior and switching in the drive electronics all produce random disturbances spread across a wide frequency range. This is broadband noise. It cannot be described by an amplitude at a single frequency, so it is characterized statistically by a power spectral density S(f). The root-mean-square load over a band follows from its integral:

Frms=f1f2SFf df	(34)

Broadband noise has no deterministic cause that can be modeled from a drawing. It is measured on the finished hardware. For well-balanced rotors in good bearings it is generally small compared with the discrete disturbances, but in very quiet systems, or between the discrete lines of a spectrum, it can set the noise floor.

### 2.7 Gyroscopic effects

A spinning rotor carries angular momentum along its spin axis:

H=Izz Ω z	(35)

If the structure it is mounted to rotates at a rate ωs about a different axis, the direction of that angular momentum is forced to change, and the rotor resists with a gyroscopic moment on its mount:

Mgyro=− ωs×H	(36)

Gyroscopic moments are therefore not a vibration of the rotor itself but an interaction between the rotor and the motion of whatever carries it. On a fixed base, ωs is zero and so is the moment. On a spacecraft that slews, or a platform that tilts, it can be large. For several rotors, the moment depends on their total angular momentum, so rotors that spin in opposite directions with equal momentum largely cancel it:

Htotal=i​Izz,i Ωi z≈0	(37)

Gyroscopic effects also change the dynamics of the rotor on its bearings. A spinning rotor’s rocking modes split into a forward whirl that stiffens with speed and a backward whirl that softens, which moves the speeds at which a disturbance can excite a resonance. This matters mainly for the structural analysis rather than for the source loads themselves.

### 2.8 Speed scaling and combining sources

Imbalance loads grow with the square of the speed. Bearing loads well below the rotor-bearing natural frequency also grow roughly with the square of speed, through Equation (28), but are recalculated at each speed rather than scaled. An imbalance load L known at one speed can be found at any other:

LN=LrefNNref2	(38)

Loads from separate sources add by superposition. When the phases between them are unknown, adding their peak values gives an upper bound on the combined load:

Ltotal≤i​Li	(39)

### 2.9 Summary of sources

| Source | Main loads | Frequency | Size predictable from design data? |
| --- | --- | --- | --- |
| Static imbalance | Fx, Fy | Once per revolution | Yes, from mass and eccentricity |
| Couple (dynamic) imbalance | Mx, My | Once per revolution | Yes, from the balance specification |
| Bearings | Fx, Fy, and Fz for angular contact bearings | Fixed non-integer multiples of shaft speed | Frequencies yes; amplitudes from geometry, preload and specified imperfections |
| Motor reaction torque | Mz | During speed changes | Yes, from spin inertia and ramp time |
| Motor ripple and cogging | Mz | Multiples set by pole and slot counts | Frequencies yes; amplitudes from datasheet or test |
| Broadband noise | All components | Spread across a wide range | No, measured only |
| Gyroscopic effects | Mx, My | Follows motion of the mount | Yes, if the mount’s motion is known |

### 2.10 How induced loads are characterized in practice

The loads a machine passes into its surroundings depend partly on the machine and partly on what it is attached to. A soft mount lets the machine move, and a mount with a resonance near an operating frequency can amplify the loads. To keep the machine’s own contribution separate, induced loads are normally defined as if the machine were bolted to a perfectly rigid base. These are called hard-mounted or blocked loads. They can be handed to any structural model, which then adds its own stiffness and damping to find the response, and they are also how hardware is tested: rotating units are mounted on a stiff force and moment table that records the six interface loads.

Before hardware exists, the loads are estimated analytically. The imbalance terms can be predicted from mass properties and balance requirements with good confidence, and bearing and motor frequencies can be predicted exactly from geometry. The sizes of the bearing, motor and broadband disturbances cannot be predicted reliably and are added once test data is available. A widely used way of describing measured rotor disturbances is the empirical model developed for reaction wheels by Masterson, Miller and Grogan, in which each load is a sum of harmonics of the spin rate:

Ft=i​Ci Ω2sin2πhifrt+ϕi	(40)

Here hi is the harmonic number of each line, Ci its amplitude coefficient and φ_i its phase. The fundamental, h = 1, is the imbalance, with C equal to Us for the force or Ud for the moment. The other lines come from bearings, motors and other imperfections, and their coefficients are fitted from test data.

A complete assessment moves from source loads, to refinement with supplier and test data, to a structural response analysis, to verification by test. This paper covers the first stage.

## 3. System Description

The assembly has three rotating parts arranged along a common line: a first momentum wheel, a rotating arm, and a second momentum wheel. Each turns on its own stationary shaft and is supported by a pair of tapered roller bearings mounted back to back. The outer race of each bearing, the cup, turns with the rotating part, while the inner race, the cone, is fixed to the shaft. Each part has its own drive motor.

The arm is a long, slender body whose length runs perpendicular to its shaft. It turns about the shaft axis through complete revolutions, behaving like a seesaw that keeps going around rather than rocking back and forth. Because it is long and narrow, its inertia is very different about its two transverse axes, which matters for its dynamic imbalance.

The two momentum wheels spin in the opposite direction to the arm and at a higher speed. Their purpose is to cancel the arm’s angular momentum, so that the assembly as a whole does not exchange momentum with its mount when the parts speed up or slow down. The wheel speeds therefore track the arm speed at a fixed ratio.

All three parts follow the same operating profile: a spin-up from rest to a hold speed, a period at constant speed, and a spin-down back to rest. The imbalance and bearing loads are largest during the hold, at maximum speed. The reaction torques from the motors act only during the spin-up and spin-down.

## 4. Coordinate System and Load Definitions

### 4.1 Reference frames

Two frames of reference are useful when describing induced vibration. The rotor frame is attached to the spinning part and turns with it. In this frame an imbalance is fixed: the heavy spot and the tilt always point in the same direction. The structure frame is attached to the mount and does not turn. This is the frame in which the structure feels the loads, and it is the frame in which induced loads are always reported.

Both frames are right-handed. By convention the z axis is placed along the spin axis, with x and y perpendicular to it. Positive rotation about z follows the right-hand rule, and a rotor turning the opposite way is given a negative spin rate. Loads are defined as those applied by the rotating machine onto its mount.

### 4.2 The six load components

Any set of loads that a machine applies to its mount can be reduced to three forces and three moments at a chosen point, written as a single six-component load vector:

d=FxFyFzMxMyMzT	(41)

| Component | Direction | Common name | Typical sources |
| --- | --- | --- | --- |
| Fx, Fy | Perpendicular to the spin axis | Lateral or radial forces | Static imbalance, bearings |
| Fz | Along the spin axis | Axial force | Angular contact bearings, axial harmonics |
| Mx, My | About axes perpendicular to the spin axis | Rocking moments | Couple imbalance, lateral forces acting at a distance |
| Mz | About the spin axis | Torque or twist | Motor reaction torque, torque ripple, cogging |

### 4.3 Reference point and load transfer

Forces are the same at every point of a rigid body, but moments depend on where they are taken. Induced loads are therefore always stated at a named reference point, usually the mounting interface where the machine attaches to the rest of the system. If a force F and a moment M act at a point P, the moment about the reference point is found by adding the moment of the force about that point:

Mref=MP+rP−rref×F	(42)

This is how loads produced at a rotor’s center of mass are moved to an interface, and how a set of loads reported at one point can be restated at another. The transfer should be made once, directly from the point where the load acts to the reference point. Moving a load in two steps, first to an intermediate mount and then to the reference point, is equivalent only if the second step starts from the intermediate point and not from the original one; otherwise the same lever arm is counted twice.

### 4.4 Several rotors and rotated axes

When a machine has more than one rotor, each rotor’s loads are first found in its own frame, with z along its own spin axis. If the spin axes are not aligned with the structure axes, each rotor’s forces and moments are rotated into the structure frame with a direction cosine matrix R before they are moved to the reference point and added:

Fstruct=R Frotor	(43)

Mstruct=R Mrotor	(44)

When all the spin axes are parallel to the structure z axis, R is the identity matrix and the loads can be added directly.

### 4.5 Rotating loads in a fixed frame

A load that is constant in the rotor frame, such as the force from a static imbalance, rotates when seen from the structure frame. Its x and y components rise and fall in turn, each reaching the same peak but a quarter of a revolution apart, and the resultant at any instant equals that peak:

Fxt=FcosΩt+ϕ	(45)

Fyt=FsinΩt+ϕ	(46)

Fx2+Fy2=F	(47)

For this reason, rotating loads are usually reported as peak amplitudes for each component. Fx and Fy are listed separately because a structural model needs both directions, but they should not be combined as if they happened at the same time, which would overstate the resultant by a factor of √2. The same applies to Mx and My. Where loads from several sources with unknown relative phase are combined, adding their peaks gives the upper bound of Equation (39).

## 5. Model Inputs

The analysis was carried out before any hardware was built, so every input comes from design data, specifications or catalogs. No balance reports, bearing vibration data or test measurements were available. The CAD models are symmetric, so on their own they show no imbalance at all. The real imbalance comes from manufacturing and assembly tolerances, which is why the eccentricity specification, rather than the CAD geometry, sets the static imbalance.

### 5.1 Inputs for each rotating part

The same set of inputs is supplied separately for each wheel and for the arm.

| Input | Symbol | Units | Source |
| --- | --- | --- | --- |
| Rotating mass | m | kg | CAD mass properties |
| Spin moment of inertia | Izz | kg·m² | CAD mass properties |
| Stored angular momentum (alternative to Izz) | H | N·m·s | Datasheet, when given |
| Static imbalance, one of: Us directly, eccentricity e, or balance grade G | Us, e, G | kg·m, µm, mm/s | Balance specification |
| Couple imbalance, one of: Ud directly, or balancing plane spacing b | Ud, b | kg·m², m | Balance specification; b from CAD |
| Maximum (hold) speed | N | rev/min | Operating requirement |
| Spin direction | ±1 | – | Design |
| Position of the center of mass along the shaft | z | m | CAD |

### 5.2 Bearing inputs

Each rotating part has its own pair of tapered roller bearings, bearing A and bearing B. Their geometry comes from the manufacturer’s catalog for the selected part number, and the imperfection amplitudes are entered separately for each bearing.

| Input | Symbol | Units | Source |
| --- | --- | --- | --- |
| Number of rolling elements | Z | – | Bearing catalog |
| Rolling element (mean roller) diameter | D | mm | Bearing catalog |
| Pitch diameter | dp | mm | Bearing catalog |
| Contact (cup) angle | α | deg | Bearing catalog |
| Effective roller contact length | l | mm | Bearing catalog or drawing |
| Axial preload on the bearing pair | Fpre | N | Assembly specification |
| Bearing radial stiffness (optional) | Krad | N/m | Supplier, when available |
| Outer race waviness, bearing A and B | Ao | µm | Bearing precision class or drawing |
| Inner race waviness, bearing A and B | Ai | µm | Bearing precision class or drawing |
| Roller waviness, bearing A and B | Ab | µm | Bearing precision class or drawing |
| Roller diameter variation, bearing A and B | Δ | µm | Bearing precision class or drawing |
| Number of harmonics included | kmax | – | Analysis setting (2) |
| Damping ratio of the rotor-bearing modes | ζ | – | Assumed (0.03) |

### 5.3 Operating and reporting inputs

| Input | Symbol | Units | Source |
| --- | --- | --- | --- |
| Spin-up time | tup | s | Motor control profile |
| Spin-down time | tdown | s | Motor control profile |
| Reference point position | zref | m | Mounting interface location |
| Other speeds of interest | – | rev/min | As requested |

### 5.4 Calculated, estimated and optional inputs

Most inputs are taken directly from CAD, specifications or catalogs. The imbalance values are specified limits rather than predictions, for the reasons given in Section 2.3. The bearing imperfection amplitudes are specified values: they should be taken from the bearing precision class or the drawing tolerances, and replaced with measured values when inspection data is available. The damping ratio is an assumed typical value.

Each imbalance can be supplied in whichever form the specification uses, and the first one given is used. Static imbalance is taken as Us directly, or from the eccentricity by Equation (5), or from a balance grade by Equation (11). Couple imbalance is taken as Ud directly or, if only a static limit exists, from the plane spacing by Equation (12). Only the spin inertia Izz is needed; the transverse inertias and the tilt of Equation (8) are not required once the imbalance is specified this way. Where a datasheet gives the stored angular momentum rather than the spin inertia, Izz follows from Equation (49) in Section 6. Any required input that is missing is reported by name and the calculation stops, so no value is silently assumed.

## 6. Model Equations

This section applies the fundamentals of Section 2 to the three rotating parts of our assembly. Every part is handled with the same equations; the subscript w is used below for a momentum wheel and the same expressions apply to the arm.

### 6.1 Speed and inertia

The operating speed of each part is given in rev/min and converted to rad/s and Hz. Where a datasheet gives the stored angular momentum Hw rather than the spin inertia, the inertia follows directly from it:

Ωw=2πNw60,  fr,w=Nw60	(48)

Izz=HwΩw	(49)

### 6.2 Imbalance loads

The vibratory loads at once per revolution come from the imbalance of each part. Static imbalance produces a lateral force and dynamic imbalance produces a rocking moment, both rotating with the part:

Fw=Us Ωw2	(50)

Mw=Ud Ωw2	(51)

where Us and Ud are the specified balance limits, entered directly or found from Equations (5), (11) and (12). Both loads grow with the square of speed, so doubling the speed quadruples them. Seen from the stationary shaft, each appears on the two sideways axes as a pair of equal sinusoids, 90° apart, as in Equations (45) and (46).

### 6.3 Moments at the reference point

These loads act at the part’s center of mass, which sits at position rcm along the shaft. Following Equation (42), they are moved directly to the reference point in a single step:

Mref=Mw+rcm−rref×Fw	(52)

Because the angle between the heavy spot and the direction of tilt is unknown, the two terms are added at full value. With L the distance along the shaft between the center of mass and the reference point, the peak rocking moment is:

Mx=My=Mw+FwL,  L=z−zref	(53)

All shafts are parallel to the structure z axis, so no rotation of axes is needed and R in Equations (43) and (44) is the identity matrix.

### 6.4 Reaction torque

The wheel and the arm also produce quasi-steady loads. Changing a part’s speed takes a torque, which the motor applies to the rotor and the stator passes into the shaft. From Equations (29) and (30), spin-up and spin-down are treated separately:

Tup=− Izz Ωwtup	(54)

Tdown=+ Izz Ωwtdown	(55)

The signs show that the mount is twisted one way during spin-up and the other way during spin-down, and parts spinning in opposite directions produce opposite signs. The net twist of the assembly is the signed sum over all three parts, which the counter-rotating wheels bring close to zero.

If the support structure rotated, each part would also apply a gyroscopic moment, Equation (36). Because all shafts are stationary, ωs is zero and this moment does not arise. The tangential imbalance force of Equation (31) is also neglected, since it is under 1% of the main force for the ramp times used.

### 6.5 Bearing loads

Each bearing of each pair is modeled separately. The bearing frequencies follow from Equations (13) to (16), using the outer-race-rotating forms, with fr taken as that part’s own rotor frequency. The preload gives the roller load and stiffness through Equations (19) to (21), and the bearing radial and axial stiffness through Equations (22) and (23). If the supplier gives a radial stiffness, it is used instead and the roller stiffness is found from Equation (22).

The rotor sits on both bearings of its pair, so its natural frequencies for lateral and axial motion are:

fn,lat=12π2Kradm,  fn,ax=12π2Kaxm	(56)

For each bearing and each harmonic k = 1 to kmax, the specified imperfections give the following disturbance lines, with fc, fo, fi and fb the cage, outer race pass, inner race pass and roller spin frequencies:

| Source | Direction | Frequency | Blocked force |
| --- | --- | --- | --- |
| Inner race waviness, q = kZ ± 1 | Lateral | k fi | Z kr Ai cos α |
| Inner race waviness, q = kZ | Axial | k fi | Z kr Ai sin α |
| Outer race waviness, q = kZ + 1 | Lateral | k fo + fr | (Z/2) kr Ao cos α |
| Outer race waviness, q = kZ − 1 | Lateral | ¦k fo − fr¦ | (Z/2) kr Ao cos α |
| Outer race waviness, q = kZ | Axial | k fo | Z kr Ao sin α |
| Roller waviness, q = 2k | Lateral | 2k fb ± fc | nb kr Ab cos α |
| Roller waviness, q = 2k | Axial | 2k fb | 2 nb kr Ab sin α |
| Roller diameter variation | Lateral | fc | kr Δ cos α |

Here nb is Z when the waviness of all rollers is assumed in phase (worst case) or √Z when their phases are taken as random. Each blocked force is converted to the force at the shaft with Equation (28), using the lateral or axial natural frequency. Lines from bearing A and bearing B fall at the same frequencies and are added. Within each rotor, all lateral lines are added to give the bearing Fx and Fy, and all axial lines to give Fz:

Flat,brg=lateral lines​Fshaft,  Fax,brg=axial lines​Fshaft	(57)

Mx,brg=My,brg=Flat,brg L	(58)

Below the natural frequency the bearing force reaches the shaft as the reaction to the rotor’s motion, so it is taken to act at the rotor’s center of mass, a distance L from the reference point. The linear model is checked by comparing the largest combined imperfection with the preload deflection δ_0 of Equation (20), and a warning is given if rollers could unload.

### 6.6 Assembly totals

The six components are summed over the arm, both wheels and all bearings, following Equation (39). Forces and rocking moments are added as peak values. The twist is added with its sign, and its magnitude is reported as the net assembly twist. Loads at any other operating point follow from Equation (38), using the arm speed, since the wheels track the arm at a fixed ratio.

| Load | Arm and each wheel | Bearings |
| --- | --- | --- |
| Fx, Fy | Fw, Equation (50) | Flat,brg, Equation (57) |
| Fz | 0 | Fax,brg, Equation (57) |
| Mx, My | Mw + Fw L, Equation (53) | Flat,brg L, Equation (58) |
| Mz | Larger of Tup and Tdown, Equations (54) and (55) | 0, since bearing drag is reacted inside the assembly by the motor |

## 7. Analysis Method

### 7.1 Overall approach

The method follows directly from the fundamentals in Section 2. Each rotating part is treated as a rigid rotor and examined for the three rotor effects: static imbalance, dynamic imbalance and reaction torque. The bearings under each part are then treated as a fourth source. The loads from every source are expressed as the six components Fx, Fy, Fz, Mx, My and Mz at a single reference point and added together, as set out in Section 6.

Three decisions frame everything that follows. First, the loads are hard-mounted, following the convention described in Section 2.10, so that they can be handed to a structural model without double counting the structure’s response. Second, the results are peak amplitudes at the maximum operating speed, which is where imbalance loads are largest. Third, because the relative phases between sources are unknown, every peak is added as if it occurs at the same instant. Real loads will be equal to or lower than the reported values. Together these choices give an answer that is conservative, easy to check by hand, and in the form most useful at the early design stage.

All three parts are handled by the same equations and the same code. Nothing in the method is specific to wheels or arms, so it applies equally to any number of rotors on parallel shafts, provided the inputs for each are supplied.

Of the sources described in Section 2, the method includes those that can be estimated from the design data available: static imbalance, dynamic imbalance, motor reaction torque and the bearings. The remaining sources are left out for specific reasons. Motor ripple and cogging need the motor pole and slot counts and ripple data, which were not yet available. Broadband noise has no deterministic model and can only be measured. Gyroscopic moments are zero because all three shafts are stationary, and even if the assembly were carried on a moving base, the counter-rotating wheels are sized to bring its total angular momentum close to zero, as in Equation (37). Each of these can be added later without changing the rest of the method.

### 7.2 Static imbalance from the eccentricity specification

Because the CAD models are symmetric, the static imbalance is taken from the balance specification rather than from the geometry: as Us directly, from the eccentricity with Equation (5), or from a balance grade with Equation (11). The lateral force at maximum speed follows from Equation (50). This force rotates with the part. It appears at the reference point as Fx and Fy, each with the same peak value, reached a quarter of a revolution apart. It produces no axial force. Whichever form is used, the rest of the method is unchanged.

### 7.3 Couple imbalance from the balance specification

An earlier version of the method calculated couple imbalance from the transverse inertias and an assumed tilt of each part on its shaft. That approach was dropped. The tilt has no specified value, and estimating it introduced a number with no traceable source, while the transverse inertias were needed for nothing else.

Couple imbalance is now treated the same way as static imbalance, as a specified limit. When a couple limit is given, it is used directly. When only a static limit exists, the couple limit is taken as that static tolerance applied in each of the two balancing planes with the residuals opposed, Equation (12), which needs only the plane spacing b from CAD. This is a stated assumption, and it is reported as such in the results. The analysis can also be used the other way round: given an allowable rocking moment M at the interface, the couple limit the balancing must achieve is Ud = M / Ω².

### 7.4 Moments at the reference point

Each part’s loads are moved from its center of mass to the reference point in a single step, using Equation (52), so that no lever arm is counted twice. Moving the reference point changes the moments but not the forces, which lets the same calculation report loads at whichever interface the structures team needs.

### 7.5 Twist during spin-up and spin-down

The twist about the shaft was calculated separately for spin-up and spin-down, Equations (54) and (55), because the two ramps can have different durations. The load tables carry the larger of the two values for each part. When the twists of all three parts are added with their signs, the counter-rotating wheels largely cancel the arm, which is the purpose of the wheels. The net value is reported as the assembly’s twist, but each motor mount still carries its own full torque, and those values are reported separately so that the individual mounts can be checked.

### 7.6 Bearings

The bearing model has two parts with different levels of confidence. The frequencies are exact. For each part, the catalog geometry gives the cage, outer race pass, inner race pass and rolling element spin frequencies from Equations (13) to (16), and these are reported in full because they are what a structural analyst needs to check for resonance.

The amplitudes follow from physics, but they depend on the imperfections specified. An earlier version of the method estimated each bearing harmonic as a fixed fraction of the imbalance force and treated each bearing pair as a single source with four harmonics. That approach was replaced. The current model builds the bearing stiffness from the catalog geometry and the preload, uses the classical waviness selection rule of Equation (24) to find which imperfection orders produce a net force and at which frequency, and passes each blocked force through the rotor-bearing dynamics of Equation (28). Each bearing of a pair is treated separately, with its own imperfection amplitudes, and each source produces its own set of lines at its own frequencies, rather than a single lumped set of four.

This makes the bearing loads traceable: every line can be followed back to a specific imperfection on a specific bearing, and the effect of a tighter tolerance class or a higher preload can be seen directly. Its accuracy rests on the imperfection amplitudes. Precision-class tolerances give a defensible starting value; inspection data on the actual bearings, or supplier vibration data, should replace them when available.

### 7.7 How the method was developed

The work started from the task itself, which was to estimate the forces and moments induced by the two wheels, the arm and the bearings, using analysis alone. The first step was to identify every mechanism by which a rigid rotor on a fixed shaft can load its mount. Working from Equations (1) and (2) showed that there are only three, and that the bearings form a fourth, separate source. This gave a complete list of sources before any equations were written, and it made clear which effects could be calculated from design data and which could not.

The reference material on reaction wheel disturbance modeling was then reviewed to confirm that the same sources and the same speed-squared scaling are used in practice. It also showed how measured data is normally folded in later, which shaped the decision to keep every estimated quantity as a single, visible input that can be replaced.

The method was then built as a script, kept deliberately short so that every line could be followed and checked. It was compared with a separate, much larger set of time-domain models covering the same hardware. Those models used the same equations for every effect included here, and the additional terms they carry are either negligible for our ramp times, zero for stationary shafts, or dependent on test data we do not yet have.

An independent review of an early version raised several points about how results could be misread. In response, the output now states that Fx and Fy are peaks reached a quarter turn apart rather than simultaneous values, labels every result as a worst-case estimate, identifies which inputs are estimates, and reports spin-up and spin-down twist separately. The built-in checks are also described as checks on the arithmetic, not on the physical inputs. Among them are the bearing identities of Equations (17) and (18), which hold for any bearing geometry, a roller-by-roller check of the bearing stiffness of Equation (22), and a numerical check of the waviness selection rule of Equation (24).

### 7.8 Implementation

The method is implemented in a single MATLAB script. The only part a user edits is the input section, where each rotating part has its own block of clearly named values. Static and couple imbalance can each be entered in whichever form the balance specification uses, and any required value left blank is reported by name before the calculation runs. When the script runs, it calculates every source, sums the loads, runs a set of arithmetic checks, plots the lateral and axial force against speed, and prints the results in four parts followed by the loads at other speeds. The flow is shown in Figure 1.

Figure 1. Calculation flow of the analysis method.

## 8. Assumptions and Limitations

The method rests on a small number of assumptions, each chosen either because it is the standard convention for source loads or because it errs toward a larger, safer answer. The parts are treated as rigid and the mount as perfectly stiff, which is the hard-mounted convention. All peaks are added as though they occur together, since the phases between sources are unknown. Fx and Fy are peak values of the same rotating force, reached a quarter turn apart, so the resultant lateral force equals Fx rather than √2 times Fx, and the same is true of Mx and My. Speed changes are taken to occur at a constant rate, and the shafts do not move in space, so there is no gyroscopic load from motion of the base.

The imbalance values are specified limits, not measurements. Where no couple limit is specified, it is taken as the static tolerance in two planes with the residuals opposed, which should be confirmed or replaced by a couple requirement. The bearing loads are only as good as the specified imperfection amplitudes, which come from tolerance classes rather than measurement. The bearing model is linear about the preload and assumes every roller stays loaded. It uses Palmgren’s empirical line-contact relation for stiffness, includes only the rotor’s lateral and axial modes on its bearings (not its rocking mode), adds the lines of both bearings and all sources in phase, and assumes a typical damping ratio.

Some effects are outside the scope of the method. It does not include structural resonance, which can raise the loads well above the hard-mounted values near a natural frequency. It does not include motor torque ripple or cogging, imbalance harmonics above the running speed, or broadband noise, and it does not model localized bearing defects such as spalls, which belong to failure analysis rather than to the induced loads of a healthy bearing. It also neglects the small tangential imbalance force during speed ramps, which is under 1% of the main force for ramps of around ten seconds.

## 9. Next Steps and Conclusion

The next steps follow directly from the estimates and exclusions above. The final CAD mass properties, speeds, positions and bearing part numbers will be entered to produce the load values for our hardware. A couple imbalance requirement will be defined or confirmed with the rotor designers, and the bearing imperfection amplitudes with inspection data or supplier vibration data. Motor torque ripple can be added once the motor pole and slot counts and ripple data are known. The loads and the bearing frequencies will be passed to the structural analysis to check for modes close to any disturbance frequency, and once hardware is built, the assembly will be measured on a force table and compared with these estimates.

Induced vibration from rotating machinery comes down to a short list of physical effects. A rotor whose center of mass is off its axis pushes sideways. A rotor whose principal axis is tilted rocks its shaft. A rotor that changes speed twists its mount. The bearings add small disturbances at frequencies set by their geometry, with sizes set by their stiffness, preload and imperfections. For our assembly, each of these was estimated from design data alone, in a form that is conservative, can be checked by hand, and can be refined one input at a time as better data becomes available.

## References

- Masterson, R. A., Miller, D. W., and Grogan, R. L. Development and validation of reaction wheel disturbance models: empirical model. Journal of Sound and Vibration, 2002.

- ISO 21940. Mechanical vibration: rotor balancing. International Organization for Standardization.

- Timken. Engineering manual for tapered roller bearings.

- Harris, T. A., and Kotzalas, M. N. Rolling Bearing Analysis. CRC Press. (Palmgren load-deflection relation, bearing stiffness.)

- Wardle, F. P. Vibration forces produced by waviness of the rolling surfaces of thrust loaded ball bearings. Proceedings of the Institution of Mechanical Engineers, 1988. (Waviness order selection rule.)

- Meriam, J. L., and Kraige, L. G. Engineering Mechanics: Dynamics. Wiley.

- Reference documents provided for this task on induced vibration and disturbance modeling.